Quantum

Optimization for

in-core fuel management

Fuel loaded into the core of Vogtle Unit 2 in Waynesboro, GA

The reactor core contains 37 cells, each of which must be filled with exactly one fuel assembly—either fresh or once-burned [1]. Same-type fuel assemblies (especially fresh-fresh) must not be placed adjacent to each other, to prevent localized power peaks and ensure neutron flux uniformity [2]. The core configuration must aim to maximize reactor power while satisfying all physical and safety constraints.

The total number of fresh and once-burned assemblies in the core must exactly match the available fuel inventory.

To ensure the physical feasibility and operational safety of the reactor core, the following constraints must be satisfied:

1-Single Assembly per Cell
Each of the 37 core positions (cells) must be occupied by exactly one fuel assembly. No cell may remain empty or contain multiple assemblies

2-No Same-Type Adjacency
Placing same-type fuel assemblies—especially fresh-fresh—adjacent to one another can result in localized power peaks and non-uniform neutron flux

3- Fuel Inventory Consistency
The total number of fresh and once-burned fuel assemblies placed in the core must exactly match the available inventory.

Solution& Pipeline

We will try to solve this this problem by Quantum annealing we will Follow this

Pipeline

Math Module	               QUBO Format                    Algorithm 	Solution

The Evaluation will be between the time of Brute force search and Quantum annealing

1st Step of Pipeline Math module

Parameters

N: Total number of core cells

i,J:  Indices that label individual cells. j is used only when talking about a cell that is adjacent to iii.

K: 0	 burnt

K: 1 	fresh fuel

____________________________________________________________________________________

1st Constraints (Single Assembly per Cell)

Constrain (Fuel Inventory Consistency)

____________________________________________________________________________________

Objective Function

We seek to minimize that bill—ideally driving it to 0 when the lattice and the specified inventories make a perfect alternation possible.

2nd step of Pipeline Building the QUBO model

The QUBO module will be built by jij Library [3]

______________________________________________________________________________

3rd Step of Algorithm

______________________________________________________________________________

Quantum annealing is a specialized quantum computing technique focused on finding the optimal solution to optimization problems. It achieves this by leveraging quantum mechanics to explore the solution space more efficiently than classical methods

4th Step Solution

We want to test our approach on 2 cases,

Cases One

Fresh. = 15, burn =22

OpenJij best raw energy: 3.00

Case two

Fresh. = 16, burn =21

OpenJij best raw energy: 0.00

Evaluation

Compare Between the Classical algorithm and quantum annealing

Brute Force Search

A literal brute-force search would have to examine every possible placement
of 10 burnt + 27 fresh assemblies in 37 cells

Quantum Annealing

For a configuration with 16 burnt and 21 fresh assemblies:  Sampling: 10,000 reads

Runtime: 15.375 seconds

Quantum annealing significantly reduces computation time by sampling from a low-energy solution space rather than exhaustively exploring all possibilities.

Reference & Code

[1] Galperin A, 1995. Nuclear Science and Engineering 119, 144–152.

[2] Usmanov, S. R., Salakhov, G. V., Bozhedarov, A. A., Kiktenko, E. O., & Fedorov, A. K. (2023). Quantum and quantum-inspired optimization for an in-core fuel management problem. Russian Quantum Center, Skolkovo, Moscow. arXiv:2308.13348 [quant-ph].

[3]Jij Inc. (2025). OpenJij: A framework for Ising and QUBO optimization via simulated annealing (Version 0.10.7) [Computer software]. https://www.openjij.org

Link for the:

Github Code